# Qcells CORE Pro Installer Application: Technical Design Document

# 1. System Overview

## 1.1 ESS State-Driven Installation Workflow

The installation workflow is split into 3 stages: server pre-sync → on-site offline configuration → post-installation sync

- Stage 1. Pre-Sync

  - After login, receive today's work orders based on the installer ID

  - Download Serial No, NMI No, model info, and the latest firmware binary of target ESS units to local storage

  - Subsequent stages proceed regardless of on-site internet connectivity

- Stage 2. Offline-First Commissioning

  - All configuration I/O performed over a BLE 1:1 direct link between the app and the device

  - Operates on the device's own data without server round trips

  - Firmware update performed over 1:1 communication with the ESS built-in Wi-Fi

- Stage 3. Completion & Cloud Sync

  - Device state confirmed as Incomplete → Run at the moment of Apply

  - Change history stored as in-app Transactions with unique IDs

  - Change history automatically synced to the cloud when internet connectivity is restored

Figure 1. ESS Installation Workflow

## 1.2 Core ESS State Definitions

All screen permissions and editable scope in the app are determined by the following 4 states

| State | Definition | Field Meaning | App Permission Policy |
| --- | --- | --- | --- |
| Incomplete | Factory reset state, configuration not completed | New installation target | Password bypass, full configuration editing allowed |
| Run | In commercial operation after configuration | Maintenance / reconfiguration target | Status view only, temporary password authentication required to configure |
| Warning / Error | Device self-diagnosis warning or halt | Field troubleshooting target | Diagnostic data first, configuration changes follow operating policy |
| Wifi / non-Wifi | Whether the device has external internet access | Whether cloud remote monitoring is available | Shows local storage sync pending when non-Wifi |

- Wifi / non-Wifi is an independent connectivity state combined with the states above (e.g. Run + non-Wifi = operating normally but not reporting to the cloud)

- FW Updating is an app session state, not a device state, returning to Run on the new version on Commit or on the previous version on Rollback

Figure 2. ESS State Transition Diagram

## 1.3 Confirmed Behaviors and Assumptions

Defines 7 field behaviors confirmed by reverse-engineering the UX spec and their design interpretation

① What the Dashboard List Really Is

- Not a live BLE scan list, but today's work list downloaded/synced based on the installer ID

- Design: Dashboard rendering fully decoupled from BLE state, offline mode guaranteed via local storage

② Install Wizard (+) Button

- Manual entry point dedicated to exceptions not covered by the download

- Cases: visits to households missing from the system, mismatches caused by on-site hardware replacement

- Installer manually enters or barcode-scans the Serial No of the physical device

- Design: Entered Serial No used as the BLE Local Name matching key to establish a 1:1 direct link, entry history recorded to the server

③ UI Lock on Detail Entry and Refresh

- BLE 1:1 connection and real-time Bulk Read occur simultaneously when an ESS Row is tapped

- All UI interaction forcibly blocked with a modal during refresh to prevent command collisions

- Device internal data force-refreshed in real time over a local BLE / socket direct link, not a server call

- Design: Session-scoped single Command Queue and Modal Lock Guard (see 2.1)

④ Permissions in Incomplete State

- Factory reset state has no password to protect, so the Temporary Installer Password step is fully bypassed

- Sequence: set the Off-Grid safety interlock at the top → enter NMI No at the bottom → Apply

- On Apply, the final [Run] state is confirmed and the device switches to security Lockdown mode

- Design: Apply treated as an irreversible transaction, mandatory confirmation dialog and settings summary screen

⑤ NMI No Cannot Be Re-entered After Deletion

- Deleting an existing NMI No on the edit screen of an uninstalled unit causes a Runtime Lockout that blocks manual re-entry

- Cause: NMI No is not a value the installer creates, but a unique asset issued and looked up via the server / settlement network (Read-Only Subscribed Asset)

- Design

  - NMI No field redefined as a Lookup Picker instead of free text input, with a "Re-fetch" action instead of delete

  - NMI No preloaded into work orders during pre-sync to prepare for offline conditions

⑥ Energy Policy and Pricing Menu Linkage

- In normal Standby mode, no tariff table is needed, so access to the Pricing submenu is fully blocked

- Pricing input form (manual per-kWh rate) unlocked only at runtime when switched to [Time-based Mode] in the top settings

- Design: Pricing availability determined by the device's own data, not local UI state

⑦ Hardware Master Switch Definitions

- Off-Grid

  - Safety unlock switch deciding whether to run standalone on solar and battery instead of halting with an error when the external grid is lost

  - Misconfiguration directly affects human / equipment safety, so a 2-step confirmation UI is applied

- Test Site

  - Whether the unit is an experimental asset for communication testing before commercial operation, when Yes the backend excludes its metering data from the general billing statistics pipeline

  - Warning banner shown just before Run confirmation to prevent forgetting to switch to No for commercial operation

## 1.4 Key Error and Recovery Paths

Every error uses device Read-back as the final source of truth, so no job ends with the app and device out of sync

| Situation | Detection | Recovery |
| --- | --- | --- |
| BLE drop during Bulk Read | didDisconnectPeripheral, pending Continuations thrown | Release UI Lock, auto-reconnect once, then offer retry to the installer |
| Write response Timeout | No Write With Response ack | No automatic resend, reconnect and compare Read-back, resend with a new OTP only if not applied |
| OTP verification failure | Device error code | Check device clock and prompt re-entry, show remaining Lockout time after consecutive failures |
| Interrupted Apply | Local incomplete marker saved before Apply | On re-entry read device state, mark complete if Run, re-Apply after settings summary if Incomplete |
| App killed / device power loss during Flash | FW session marker found on relaunch | Read device version, Commit if new version, confirm Rollback and retry if previous version |
| SoftAP join failure | NEHotspotConfiguration error | Retry once, then request BLE Wake-up again and reissue SSID |
| Install Wizard scan failure | No Local Name match within Timeout | Re-check Serial No, guide installer closer (RSSI shown), rescan barcode |
| Cloud Sync failure | HTTP 5xx / network error | Retry with increasing Timeout, treat 409 duplicate as success |

# 2. Mobile Architecture and Core Interfaces

## 2.1 Layered Information and Clean Architecture

UI / Domain / Data 3-layer separation isolates hardware spec changes from propagating to the UI

Figure 3. System Context Diagram

| Layer | Components | Responsibility |
| --- | --- | --- |
| UI Layer | SwiftUI View, @MainActor ViewModel | Screen rendering, UI Lock display, forwarding user actions |
| Domain Layer | UseCase, Entity, ESSStateMachine | State transition rules, permission policy, Pricing availability |
| Data Layer | Repository, BLETransport, SoftAPTransport, REST Client, Local Store | Device register I/O, server communication, local persistence |

- Dependency Direction

  - UI → Domain ← Data (Domain owns only Protocols)

  - BLETransport defined as a Protocol so Mock / real implementations can be swapped

- Dashboard Refresh Spec

  - Pull-to-Refresh re-syncs the work list with the server

  - Home detail screen Refresh receives device data (BLE communication)

- Modal UI Lock Guard Spec

  - Screen blocked with fullScreenCover while device data I/O is in progress

  - .allowsHitTesting(false) and .interactiveDismissDisabled() applied to child views

  - Lock released with a retry option after TBD seconds Timeout in case of device unresponsiveness

  - Session-level exclusive access via actor DeviceSession, duplicate commands within the same session rejected at the Command Queue

protocol BLETransport {

var connectionState: AnyPublisher<ConnectionState, Never> { get }

func connect(serialNo: String) async throws

func read(_ register: Register) async throws -> Data

func write(_ register: Register, value: Data) async throws

}

## 2.2 Platform-Compliant Hardware-Backed Protection of Sensitive Data

Sensitive data is stored only in the Keychain, and BLE commands are serialized through an actor-based queue for thread safety

| Data | Storage | Accessibility / Access Control |
| --- | --- | --- |
| Installer Auth Token | Keychain | kSecAttrAccessibleWhenUnlockedThisDeviceOnly + .biometryCurrentSet |
| Refresh Token | Keychain | kSecAttrAccessibleAfterFirstUnlockThisDeviceOnly |
| Temporary Installer Password | Memory only | Never persisted, discarded immediately after use |
| Work Order Data | Local DB | Data Protection .complete |

- Security Principles

  - ThisDeviceOnly attribute blocks iCloud Keychain sync and backup restore

  - Token automatically invalidated by .biometryCurrentSet when biometrics are re-enrolled

  - Serial No, NMI No, and passwords masked in logs / Crash Reports

- Async-Await Based BLE Communication

  - CoreBluetooth Delegate Callbacks converted to CheckedThrowingContinuation

  - Pending Continuation limited to 1 per Characteristic to prevent double Resume

  - All pending Continuations thrown at once on Disconnect

- State Streams and Thread Safety

  - Connection state, RSSI, and real-time metering published via Combine CurrentValueSubject, command execution path separated from observation path

  - CBCentralManager runs on a dedicated serial DispatchQueue, GATT Writes serialized through an actor-internal queue

# 3. Resilient Configuration Detailed Design

## 3.1 Model-Specific Service UUID Targeted Scan and Overcoming Wireless Constraints

Models are filtered first by Service UUID, then identified 1:1 per Serial No via Local Name matching

- Model-Specific Service UUID Hardware Filtering

  - Model-specific Service UUIDs passed to scanForPeripherals(withServices:), eliminating noise from unrelated nearby BLE devices

  - Per-model UUID / Local Name format TBD (e.g. AACCB-{SerialNo}), updated via server Remote Config when models are added

- iOS CoreBluetooth MAC Address Virtualization Constraint

  - iOS does not expose physical MAC addresses to apps

  - CBPeripheral.identifier is a randomly generated UUID per iPhone, so the same device cannot be identified across different iPhones

- Advertising Data Local Name String Matching Workaround

  - Serial No extracted from advertisementData[CBAdvertisementDataLocalNameKey] and compared with the work list or manual input

  - When shortened due to advertising length limits, confirmed by reading Serial Number String (0x2A25) after Connect

- Scan Operation Policy

  - Foreground-only scanning, stopScan() called as soon as the target is confirmed to minimize battery drain

  - When multiple devices respond with the same Serial No, the highest RSSI is selected and the installer is asked to confirm

## 3.2 Offline Firmware 5-Stage Lifecycle Triggered by BLE Wake-up Command

Wi-Fi is not kept running, the built-in Wi-Fi starts only while requested via a BLE command

- Design Background

  - Always-on device Wi-Fi expands the external attack surface, and BLE bandwidth alone is too slow for multi-MB firmware transfers

  - Conclusion: BLE for control, temporarily started Wi-Fi for bulk transfer

- Dynamic Wi-Fi Trigger Sequence

  - App → Device: send BLE Wake-up Command

  - Device: generate one-time SSID / Passphrase, start built-in Wi-Fi, return connection info over BLE

  - App: join the Wi-Fi via NEHotspotConfiguration (joinOnce = true)

  - Device automatically shuts down Wi-Fi on transfer completion or after TBD minutes Timeout

  - Required iOS permissions: Hotspot Configuration Entitlement, Local Network permission

- Firmware 5-Stage Lifecycle

| Stage | Device Action | App Action | Failure Handling |
| --- | --- | --- | --- |
| 1. File Transfer | Receive binary into Inactive Bank | Send pre-synced binary in chunks, show progress | Resume from interruption point |
| 2. Integrity Check | Checksum / MD5 comparison, Secure Boot signature verification | Read verification result over BLE | Discard Inactive Bank on failure |
| 3. Flashing | Write to Inactive Bank of Dual-Bank Flash Memory | Keep UI Lock, prevent screen sleep (isIdleTimerDisabled) | Active Bank kept intact |
| 4. Hardware Restart | Switch Boot Bank and Reboot | Wait for BLE reconnect and rescan | Automatic Rollback to previous Bank on boot failure |
| 5. Version Commit | Commit new Bank as Active | Final check via version register Read-back | Retry or report failure on version mismatch |

- Precondition Guards: device-side Flash conditions (battery SOC, grid state) TBD, update blocked when iPhone battery is below TBD%

## 3.3 One-Shot Transaction Authentication and UUID-Based Cloud Post-Sync

Configuration changes on Run devices are protected by per-command one-time authentication, and cloud updates are applied without duplicates via unique UUIDs

- Authenticated One-Shot Write

  - Packet structure: [Register ID][Parameter Payload][OTP Suffix (Time-based)]

  - When changing settings in Run state, a time-based OTP token derived from the Temporary Installer Password is appended as a suffix and sent as a single packet

  - Device applies only that one command when OTP verification succeeds, no session-level permission retained

  - Result confirmed by reading back the same register right after the Write, OTP suffix omitted in Incomplete state

- OTP Rules

  - Time Step TBD seconds, tolerance ±1 Step, temporary device-side Lockout after TBD consecutive failures

  - Device clock checked when reading device data, considering device conditions at remote sites

- UUID-Based Cloud Post-Sync

  - Unique UUID issued when a configuration succeeds, changes persisted in local storage (retained across app termination / reboot)

  - Changes uploaded sequentially when NWPathMonitor detects internet connectivity is restored

  - Server returns the existing result without re-applying when the same UUID is received again

  - Retries with increasing Timeout on failure, unsynced status shown in the Dashboard list

- Restart Recovery

  - Transactions tracked as Pending → Applied → Synced (or Failed), recorded as Pending just before the Write is sent

  - On relaunch, Pending entries are resolved to Applied / Failed by Read-back comparison on the next device connection

  - Applied entries upload automatically when the network returns, re-uploads never double-apply thanks to the same UUID

## 3.4 Backend and Firmware Interface Contract (Assumed)

Timeout, Retry, and failure handling are fixed per command for both device commands and server APIs, making field behavior predictable

- Device Commands / Events (values finalized once the hardware spec is available)

| Command / Event | Channel | Timeout | Retry | Failure Handling |
| --- | --- | --- | --- | --- |
| Connect + Version Handshake | BLE | TBDs | 2x after rescan | Proximity guidance, re-check Serial No |
| Bulk Read | BLE | TBDs | 1x | Release UI Lock, offer retry |
| One-Shot Write | BLE | TBDs | No automatic resend | Decide after Read-back comparison |
| Wake-up (start SoftAP) | BLE | TBDs | 1x | Request SSID reissue |
| FW Chunk Transfer | SoftAP | TBDs per chunk | TBDx per chunk | Resume from interrupted offset |
| FW Commit / Version Check | BLE | Reboot wait TBDmin | Rescan | Check for Rollback |
| State Change Event | BLE Notify | N/A | N/A | Reflect Warning / Error on screen immediately |
| Flash Progress Event | BLE Notify | Silent TBDs | Reconnect | Enter FW session recovery |

- Backend APIs

| API | When Called | Error Handling |
| --- | --- | --- |
| Work Order fetch | Pre-sync after login | On failure use last local cache and show sync time |
| NMI Lookup | Pre-sync, re-fetch when online | Offline allows preloaded values only |
| Installer Password issuance | Pre-sync (per job) | Prompt online reissue when expired |
| Firmware binary + metadata | Pre-sync | Re-download on checksum mismatch |
| Transaction Sync | When network returns | 401 refreshes Token, 409 treated as success, 5xx backs off |
| Exception entry log | After Install Wizard use | Sent through the Outbox like Transactions |

- Version Compatibility

  - Read the device Protocol Version right after connecting and compare with the app's supported range (Min ~ Max)

  - Device too old: block configuration Writes and prompt firmware update, device too new: require app update

  - Register Map differences by version managed via a Capability table in server Remote Config, no app release needed

  - Firmware metadata includes supported HW Revision and mandatory intermediate versions to block skip-updates

  - Backend APIs pinned by URL version (/v1), forced app update below the minimum app version

# 4. Verification and Testing Approach

## 4.1 Emulation and Virtual Operation (Mock & Simulators)

A register-level Mock is built to automatically verify state transitions and UI constraints without hardware

- Mock Peripheral Design

  - Mock implementation of the BLETransport Protocol holds a virtual device data Dictionary

  - iOS Simulator does not support real BLE, so the full flow runs with the Mock injected, swapped via the -UITestMockBLE launch argument

- Device Data Protocol Mocking Examples

  - Reproduces Energy Policy Standby → Time-based transition responses, and Incomplete → Run transition followed by OTP-required responses

  - Fault injection options for response delay, no response, and disconnection

| ID | Scenario | Expected Result |
| --- | --- | --- |
| UT-01 | Enter Pricing in Energy Policy Standby state | Menu disabled |
| UT-02 | Enter Pricing after switching to Time-based Mode | Per-kWh rate input form enabled |
| UT-03 | Touch the screen while reading data on detail entry | All input blocked |
| UT-04 | Enter an Incomplete device | Password step bypassed |
| UT-05 | Execute Apply | State changes to Run, OTP required on re-entry |
| UT-06 | NMI No Re-fetch action | Server value re-injected via Lookup Picker |
| UT-07 | Network restored after offline configuration | Single upload from persistent storage, no duplicates |

## 4.2 Real Hardware Verification Setup

Power safety behavior and wireless variables that Mock cannot verify are validated in a test environment

- Test Environment: real CORE Pro device, Grid Simulator or breaker-based grid disconnection, PV Simulator, RF Attenuator and shielded box, iPhone Device Farm (multiple models / versions)

| ID | Condition | Action | Expected Result |
| --- | --- | --- | --- |
| 1 | Off-Grid ON | Grid cut | Automatic EPS switchover, standalone on solar + battery |
| 2 | Off-Grid OFF | Grid cut | Error halt, Error state shown in app |
| 3 | EPS running | Grid restored | Grid reconnection and return to Run |
| 4 | During flashing | Device power cut | Rollback to previous Bank after reboot |
| 5 | RSSI boundary | Attenuator attenuation | Connection allowed / blocked per threshold policy |

- EPS Switchover Time Criteria: finalized as TBDms once the hardware spec is available

# 5. Trade-offs and Risk Management

## 5.1 Key Operational and Data Model Assumptions

Externalized Scheduling Assumption

- Work assignment and scheduling are owned by the HQ system, the app only consumes the work list and has no schedule editing

- Impact: reduced app scope and simpler offline handling, but on-site schedule changes depend on server re-sync

NMI No Primary Key Binding Assumption

- NMI is a unique identifier per metering point, NMI No ↔ Site ↔ ESS bound 1:1:1 and used as the Primary Key

- To verify: whether multiple ESS units are installed under the same NMI, if so switch to a composite key (NMI No + Serial No)

## 5.2 Technical Trade-offs

Limiting Real-Time Metering Polling Streams

- High-frequency polling (1s) gives real-time feel but delays configuration changes and loads the device MCU, low-frequency polling (TBDs) is the opposite

- Adopted: poll only while the metering screen is in the Foreground, pause polling while configuration commands are pending, stop immediately on entering Background

Dynamic RSSI Guard Threshold Relaxation Combined with MFA

- Rooftop / exterior-wall installations inevitably have low RSSI due to distance and shielding between installer and device

- A fixed Threshold blocks legitimate field work, while unlimited acceptance risks connecting to a neighbor's device

- Adopted: minimum signal strength relaxed step by step according to identity verification strength

| Verification Level | Verification Method | RSSI Guard |
| --- | --- | --- |
| Level 1 | Local Name matching | Default TBD dBm |
| Level 2 | Local Name + Barcode / QR scan | Relaxed TBD dBm |
| Level 3 | Level 2 + One-Shot OTP | Write allowed |

## 5.3 Key Risks and Open Issues

Risk 1. ESS Built-in Wi-Fi Failure

- Long-term non-Wifi state occurs when the built-in Wi-Fi does not work

- Mitigation: the app collects accumulated device logs over BLE during installer visits and uploads them on its behalf, the server automatically creates a revisit work order when the threshold is exceeded

- Open issue: device internal log buffer capacity and retention period

Risk 2. Mandatory NMI No for Fully Off-Grid Remote Households

- Remote households not connected to the grid may have no NMI No issued, so configuration cannot be completed under current Apply rules

- Improvement: backend auto-fills an Off-Grid-only temporary identifier, such units isolated from the settlement pipeline, server-side key replacement once an official NMI is issued

Risk 3. Missing Hardware Spec

- Service UUID, Local Name format, register map, and OTP algorithm not yet defined

- Mitigation: isolated behind the BLETransport Protocol boundary so only the Data Layer changes once the spec is finalized

Open Questions

- Does the Local Name contain the full Serial No, or a shortened form

- Who issues the Temporary Installer Password, what is its validity period, and which OTP algorithm is used

- What are the firmware binary size and the built-in Wi-Fi transfer speed baseline

- Is the authority to switch Test Site → commercial operation with the installer or with HQ

- Is changing the Off-Grid setting allowed in Run state